This document serves to describe the necessary functionality of the electronics that control the Purfect cat litter box.

# Overview

The device filters dirty air by drawing air from the cat litter box by means of a Venturi. The water circulation flow is generated with a pump that pumps from a water reservoir back into the same reservoir. The venturi is located on the high-pressure side of the pump (output). The pump is activated when motion is detected. The pump stops automatically after a certain set time. When the manual mode is selected, the pump runs continuously.
The touch buttons are:
a. 1 button. ON/OFF switching
b. 2 buttons +/-. menu scrolling and raising/lowering automatic ON-time.

c. 1 button: selection between auto/man or settings menu after long press.

# Components

- Water pump (preferably a 5VDC version)

- Venturi nozzle

- Motion detection sensor

- PCB with relay for the water pump

- 4 touch sensors for menu control

- NeoPixel LED for menu indication

- speaker

# Electric Diagram

# Shell

Location of touch buttons

Location of NeoPixel

# Specifications

- All components external to the PCB should be easily replaceable with a connector
(Touch buttons, NeoPixel, Motion Sensor)

- Power rating should be 5VDC.

- An extra USB-C should be added for the power supply since the onboard USB-C cannot provide enough current for the water pump.

- The possibility to update the device is required. (SD > Bluetooth > Wifi)
The PCB should probably have the need for an (integrated) SD card.
This would also allow for a storage of sound clips.

- The capacitive touch buttons are located on the inside of the shell.

# Extra Remarks

- The current project is based on a Seeeduino/Arduino Nano but any suggestions is welcome. Arduino was chosen for the programmability.

- Programming can be done by us if the Arduino technology is chosen. If not, when cheaper solution is provided, this document will be updated with a more extensive description of the workflow.

- If for CE approval a hardware ON/OFF (vs the soft ON/OFF), the please include it.

- The project was conceptualized with Arduino and